Interface Specification

The Camera supports UVC protocol in Linux. You need to load V4L2 module to manipulate camera. The virtual machine and HUB may report data transfer error while there are massive data transmission.

V4L2 relevant:

int init_v4l2(void)；// Initialize V4L2

int v4l2_grab(void); // Collecting Data

int v4l2_control(int); // Control Camera

int v4l2_release();// Release

Tips: Length & Width:

The last four lines of the screen are set aside for data transmission. For instance, the original resolution is 384×288; when capturing data, the resolution must set as 384×292,292-288=4, there 4 lines are set for data transferring.

Camera Manipulation and relevant settings:

Control commands of Chipset use Zoom (Absolute) control channel in UVC protocol. The commands contain two bytes: lower byte is parameter and high byte indicates register address.

For example: invoke “v4l2_control(0x8000)” when sending shutter calibration command 0x8000.

Commands:

| Serial Number | Register Address | Parameter | Description |
| --- | --- | --- | --- |
| 1 | 0x80 | 0x00 | Shutter Calibration |
| 2 |  | 0x01 | Background Calibration |
| 3 |  | 0x02 | Raw Output of Sensor |
| 4 |  | 0x04 | 16bit NUC Raw Data |
| 5 |  | 0x05 | YUYV Data Output |
| 6 |  | 0x20 | Room Temperature Measuring |
| 7 |  | 0x21 | Extended Temperature Measuring (Available for T3Pro Only) |
| 8 |  | 0xFE | Save configurations (Data source switching, palettes) |
| 9 |  | 0xFF | Save Temp. Measuring Parameters |
| 10 | 0x88 | 00-07 | Palette |

- Shutter Calibration: execute nonuniformity calibration to uniform imaging, improve measuring accuracy. Industrial measuring calibrates once every three minutes while Body temperature measuring calibrates once per minute. After 0.5 seconds of calibration (frame rate of infrared camera is 25Hz), proceed to thermometry calculation (refer to thermometry library).

2. Background Calibration: point the camera towards uniform scenes for shutter calibration.

3.Differences between 0x8004 and 0x8005:

⑴ In 0x8004 mode, even though output format is YUYV format, the actual transferred is 16bit NUC data. In every continuous 16 bits, there are 14 bits to store the wide dynamic grayscale value of one pixel. In 0x8005 mode, the output data is in YUYV format.

(2) In 0x8004 mode, after processing libthermometry.so, raw data is transformed to global temperature data. Then processed in libSimple.so or Linear Algorithms (refer to Source Code: Main1 section), temperature data will transform to rgba data. In 0x8005 status, be aware of the last 4 lines of parameters in the output yuyv data. After processing libthermometry.so, you can obtain the following data from the last 4 lines of parameters:

Temperature of center;

The highest temperature and location;

The lowest temperature and location;

Temperature of three customized points;

And you cannot obtain global temperature.

4.Room temperature range and extended temperature range:

The room temperature ranges: -20℃ to 120℃; extended temperature ranges: -20℃ to 400℃. Only T3pro supports extended temperature range. For body temperature measuring, setting varies base on the device.

5.About Palette:

(1) In 0x8005 status, switch palette via sending 0x88 (00-07). Palettes including: White Hot, Black Hot, Iron Rainbow, Lava, Rainbow, Iron Grey, Red Hot and Rainbow 2.

- In 0x8005 status, switch palette via Compute(...) or Linear algorithms. Palettes including: White Hot, Black Hot, Iron Rainbow, Rainbow 1, Rainbow 2, High Contrast Rainbow 1, High Contrast Rainbow 2.

Thermometry library relevant:

There are two steps to thermometry: calculate thermometry table; search on the table via 16bit raw data.

thermometryT and thermometryT4Line are functions for calculating thermometry table. The differences are: thermometryT uses head address of data while thermometryT4Line uses address of the last 4 lines of parameters. thermometrySearchXXX function as searching table. Check the annotation in thermometry.h for more details.

void sendCorrection(float correction);

void sendReflection(float reflection);

void sendAmb(float amb);

void sendHumidity(float humidity);

void sendEmissivity(float emiss);

void sendDistance(float distance);

void savePara();

These functions above are relevant to setting thermometry parameters. Once parameters are set, user are required to calibrate the shutter manually. Then recalculate the thermometry table, check the results and save the parameters. The set parameters may be lost without saving, so remember to save the parameters.

Image Processing relevant:

There are two image processing methods: professional image processing algorithms and Linear Algorithms. The professional algorithms require the basic frequency to be at least 1.8GHz while there are almost no requirements in Linear Algorithms. So, for thermometry, recommend using linear algorithms.

The palettes of two algorithms are in the libSimple.so library

For Professional Algorithms, refer to annotation in SimplePictureProcessing.h

For Linear Algorithms, refer to annotation in main1.cpp

How to run the Demo(main1.cpp)

Place the library that suits target platform to the same level directories of the demo file.

The OpenCV4 library is required in this demo. Type the following command in terminal to run the demo, after installation of OpenCV4:

g++ main1.cpp -L. -lthermometry -lSimple -lm -o main1 `pkg-config opencv4 --libs --cflags`

export LD_LIBRARY_PATH=./

./main1

In the demo, OpenCV performed the conversion of yuyv to rgb and output functions, which are not necessary.